Features：

- RGB color

- Common Cathode

- High bright output

- Low power consumption

- High reliability and long life

Descriptions：

- Dice material：AlGaInP

- Emitting Color：Super Bright RGB

- Device Outline：φ5mm Round Type

- Lens Type：White diffused

Directivity：

- All dimensions are millimeters

- Tolerance is +/-0.20mm unless otherwise noted

4、Absolute maximum ratings（Ta = 25℃）：

| Parameter | Color | Symbol | Test Condition | Value |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Max. |  |
| Power Dissipation | Red | Pd | —— | —— | 75 | mW |
|  | Green |  |  |  | 120 |  |
|  | Blue |  |  |  | 120 |  |
| Reverse Voltage | —— | VR | IR = 30μA | 5 | —— | V |
| Pulse Current | —— | IFp | Duty=0.1，1kHz | —— | 100 | mA |
| Forward Current | —— | IFm | —— | —— | 25 | mA |
| Operating Temperature | —— | Topr | —— | -40 | +80 | ℃ |
| Storage Temperature | —— | Tstr | —— | -40 | +85 | ℃ |

5、Electrical and optical characteristics（Ta = 25℃）：

| Parameter | Color | Symbol | Test Condition | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Typ. | Max. |  |
| Forward Voltage | Red | VF | IF = 20mA | 1.8 | —— | 2.5 | V |
|  | Green |  |  | 2.8 | —— | 3.5 |  |
|  | Blue |  |  | 2.8 | —— | 3.5 |  |
| Reverse Current | —— | IR | VR = 5V | —— | —— | 30 | μA |
| Wavelength | Red | Tc | IF = 20mA | 620 | —— | 635 | nm |
|  | Green |  |  | 520 | —— | 530 |  |
|  | Blue |  |  | 460 | —— | 470 |  |
| Luminous Intensity | Red | IV | IF = 20mA | 600 | —— | 800 | mcd |
|  | Green |  |  | 2000 | —— | 3000 |  |
|  | Blue |  |  | 700 | —— | 1000 |  |
| Viewing Angle | —— | 2θ1/2 | IF = 20mA | —— | 50 | —— | Deg. |

6.Typical photoelectricity characteristic curve chart